TPS3851H33 WATCHDOG TIMEOUT
DESIGN VERIFICATION REPORT

Updated Datasheet Equation Verification — Cases C20 and C45

Project: OpenMycoNet Environmental Telemetry & Bioelectrical Sensing Platform

Document Date: October 19, 2026

Scope: Corrected TPS3851 / TPS3851-Q1 Standard-Timing Watchdog Calculations Only

# 1. Datasheet Timing Equations

The adjustable watchdog timeout is determined by the capacitor connected to the CWD pin. The following Texas Instruments TPS3851 / TPS3851-Q1 equations are used for verification.

Standard Timing Version

Watchdog timeout (ms) = 3.23 × C_CWD (nF) + 0.381 ms

Extended Timing Version

Watchdog timeout (ms) = 77.4 × C_CWD (nF) + 55 ms

This report evaluates the standard-timing equation for the two capacitor cases used in the design discussion. Capacitor value is entered in nF and the calculated watchdog timeout is returned in ms.

# 2. Case 1 — C20 = 2.7 nF

Configuration: Proposed small CWD capacitor value for the standard-timing watchdog variant.

## Nominal Timeout Calculation

Step 1. Substitute the capacitor value into the standard-timing equation:

Watchdog timeout = 3.23 × 2.7 + 0.381

Step 2. Multiply the capacitor value by the timing coefficient:

3.23 × 2.7 = 8.721 ms

Step 3. Add the fixed timing offset:

8.721 + 0.381 = 9.102 ms

Final nominal result:

C20 = 2.7 nF  →  Typical watchdog timeout = 9.102 ms ≈ 9.1 ms

## Timing Range

| Parameter | Timeout |
| --- | --- |
| Minimum watchdog timeout | 7.34 ms |
| Typical watchdog timeout | 9.1 ms |
| Maximum watchdog timeout | 11.0 ms |

Engineering conclusion:

A nominal watchdog window of approximately 9.1 ms is too narrow for reliable ESP32-S3 operation during sensor acquisition and normal firmware execution. I2C and SPI transactions, scheduling latency, storage activity, LoRa processing, or other routine operations could prevent a timely watchdog service and trigger false hardware resets.

# 3. Case 2 — C45 = 100 nF (Current Rev 3.5 Build)

Configuration: Installed CWD capacitor value on MCU Sheet 7 for the current Rev 3.4 build, using the standard-timing watchdog variant.

## Nominal Timeout Calculation

Step 1. Substitute the capacitor value into the standard-timing equation:

Watchdog timeout = 3.23 × 100 + 0.381

Step 2. Multiply the capacitor value by the timing coefficient:

3.23 × 100 = 323 ms

Step 3. Add the fixed timing offset:

323 + 0.381 = 323.381 ms

Final nominal result:

C45 = 100 nF  →  Typical watchdog timeout = 323.381 ms ≈ 323.4 ms

## Timing Range

| Parameter | Timeout |
| --- | --- |
| Minimum watchdog timeout | 274.9 ms |
| Typical watchdog timeout | 323.4 ms |
| Maximum watchdog timeout | 371.9 ms |

Engineering conclusion:

The 100 nF C45 capacitor produces a nominal watchdog timeout of approximately 323.4 ms. This provides the ESP32-S3 sufficient timing margin to refresh the watchdog heartbeat during active sensing, I2C/SPI communications, data buffering, and LoRa activity while retaining a rapid hardware recovery response following a firmware lockup.

# 4. Final Design Decision

- Rejected case: C20 = 2.7 nF produces a 9.1 ms typical timeout and is unsuitable for the ESP32-S3 sensing application because it presents a high risk of false watchdog resets.

- Verified Rev 3.5 implementation: C45 = 100 nF produces a 323.4 ms typical timeout, with a 274.9 ms to 371.9 ms expected timing range, and remains the appropriate installed watchdog-timing capacitor for the current design.